Plantar Pressures Data Collection

Synchronizing Novel emed XL and Vicon Motion Capture

*** When finished, please put everything back to the way it was when you found it***

Set Up

Vicon
Needs to have been properly warmed up and calibrated.

Data is collected in: D > Raw Data > NovelSync

Labeling skeleton = LowerbodyClusterMultiSegFoot

*note: no model used, no cal trial needed, but take one anyway just in case

Configuration = MainLabSyncNovel

Data capture is at 100Hz

Cables

Switch the cable in the back of the middle black box from ‘Remote Start’ to ‘1’ in Sync Outputs

Check the cables below the video station, and make sure that the ACTIVE cable is connected.

In Novel:

Open patient file per standard protocol.

Then, go to

Options > port settings > configure devices

In the pop up (first screenshot) select ‘NovelSync’ under hardware & select the plantar pressure mat, ensuring that the pop up matches the second screenshot below.

Data Collection

Place foot markers. See list at end of document for picture and list of marker placements.

Capture 10 trials over the force platforms and PP mat.

We are looking for: a minimum of two full consecutive steps over the PP mat for multiple trials

Clean foot strikes on force platforms (1 left and 1 right per trial, ideally)

If this is not happening, have the participant change their start location.

Data Transfer

First, export Ascii Files by following the instructions below.

Double click on the trials you want.

Then click “Analysis”

Click emascii.

The window below (showing each trial) will pop up, select File > Save All > Ok and route to your desired save location (often C:\novel\novfile\ascii\asciiout)

The program may stop responding, but it’s okay. Give it a few minutes.

Move MoCap Files To   S:\cgma_ResearchMoCap\Projects\NovelSyncValidation

Move Ascii files

from  C:\novel\novfile\ascii\asciiout

to   S:\cgma_ResearchMoCap\Projects\NovelSyncValidation

Data Processing

Label and gap fill in Vicon.

Then, run the c3d and ascii files through the Impressions MATLAB code.

Marker Placement. For your convenience, here are some photos of the marker set up, and a list of all markers on the left side.

The set up for the right side is identical, except the first ‘L’ is a ‘R’. The markers written in green are mandatory, the markers in black optional. The black markers are listed because they are in in the labeling skeleton.

| Marker Name | Marker Location |
| --- | --- |
| SACR SCRL SCRR L/RASI L/RTHI L/RTHIA L/RTHIP L/RKNEE L/RMED L/RFIB L/RTUB L/RTIB L/RTIBA L/RTIBP L/RANK L/RMMA L/RHEEL L/RLCA L/RMCA L/RTOE L/RD1M L/RD5M L/RP1M L/RP5M L/RACH L/RHLX | Sacrum. Not placed bilaterally. Left PSIS Right PSIS Anterior superior iliac spine Thigh (serves as wand), cluster Anterior thigh, cluster Posterior thigh, cluster Lateral knee, on the lateral epicondyle Medial knee, on the medial epicondyle Head of the Fibula Tibial tuberosity Tibia (serves as wand), cluster Anterior tibia, cluster Posterior tibia, cluster Lateral Ankle, on the lateral malleolus Medial Ankle, on the medial malleolus Heel – placed even with TOE when standing flat footed Lateral calcaneus – even with HEE, placed 1 base width lateral Medial calcaneus – even with HEE, placed 1 base width medial Base of the 2nd toe on the left side Distal head of the 1st met (big toe) Distal head of the 5th met (pinkey toe) Proximal head of the 1st met Proximal head of the 5th met Achilles, one base width above the heel Hallux (big toe). Proximally and medially. |